- Connect the module to a Test computer by an USB-to-TTL tool or a MAX232 transformer

- Connect a 13.56Mhz RFID antenna to the module

- Power on the module

- Use a Terminal Tool such as AccessPort137 to send HEX command to the module,baund-rate is 115200,N,8,1

- Use command 50 00 01 21 05 75 to open the antenna

- Use command 50 00 01 21 00 70 to close the antenna

- Use command 50 00 05 23 FF 64 00 01 05 E9 set the module to auto-running